Campus network report
week2/basic connectivity and implementation.

Week 2 objective was to build and prove the basic connectivity of the campus network inside GNS3. This means creating the VLAN structure on the switch, configuring the router to handle traffic for all three VLANs, and ensuring that every PC connected to the network receives an IP address automatically through DHCP.

#### Network Design Overview

The campus network is built using a Router This means one physical cable connects the router to the switch, and all three VLANs share that single link. The VLANs are separated at the router using sub-interfaces

Three vlans assigned :

| vlan | Name | Subnet | gateway |
| --- | --- | --- | --- |
| 10 | student | 192.168.10.0/24 | 192.168.10.1 |
| 20 | staff | 192.168.20.0/24 | 192.168.20.1 |
| 99 | guest | 192.168.99.0/24 | 192.168.99.1 |

Physical Interface (FastEthernet0/0)

FastEthernet0/0 is the trunk port that connects the router to the switch. No IP address is assigned on this interface because IP addressing is handled on the sub-interfaces. The interface is simply brought up with no shutdown.

VLAN Sub-Interfaces

Three sub-interfaces are created on FastEthernet0/0, one per VLAN.

- Fa0/0.10 handles VLAN 10 (Students). It is assigned IP 192.168.10.1 and acts as the default gateway for all student devices.

- Fa0/0.20 handles VLAN 20 (Staff). It is assigned IP 192.168.20.1 and acts as the default gateway for all staff devices.

- Fa0/0.99 handles VLAN 99 (Guests). It is assigned IP 192.168.99.1 and acts as the default gateway for all guest devices.

DHCP Configuration

The router acts as the DHCP server for all three VLANs. The first 9 addresses in each subnet are excluded using the ip dhcp excluded-address command. These reserved addresses are kept for the router gateway, servers, printers, and any future network devices.

Three DHCP pools are configured:

- STUDENTS pool — serves the 192.168.10.0/24 subnet, uses Google DNS (8.8.8.8), and sets a lease time of 1 day.

- STAFF pool — serves the 192.168.20.0/24 subnet, uses Google DNS (8.8.8.8), and sets a lease time of 1 day.

- GUESTS pool — serves the 192.168.99.0/24 subnet, uses a local DNS server at 192.168.99.2 (the captive portal)

VLAN Database

Three VLANs are created on the switch using the vlan command VLAN 10 is named STUDENTS, VLAN 20 is named STAFF, and VLAN 99 is named GUESTS. Without creating the VLANs here, the switch will not forward tagged traffic for those IDs.

Access Port Assignments

Each port that connects to a PC is set to access mode using switchport mode access. The command switchport trunk allowed vlan 10,20,99 restricts the trunk to only carry the three campus VLANs, which improves security by blocking all other VLAN traffic.

Configurations :

CAMPUS ROUTER

enable

configure terminal

hostname CampusRouter

no ip domain-lookup

service password-encryption

enable secret Cyberia@2024

ip cef

PHYSICAL INTERFACE

interface FastEthernet0/0

description TRUNK-TO-SWITCH

no ip address

no shutdown

VLAN SUB-INTERFACES

interface FastEthernet0/0.10

description STUDENTS-VLAN10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

ip nat inside

no shutdown

interface FastEthernet0/0.20

description STAFF-VLAN20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

ip nat inside

no shutdown

interface FastEthernet0/0.99

description GUESTS-VLAN99

encapsulation dot1Q 99

ip address 192.168.99.1 255.255.255.0

ip nat inside

no shutdown

DHCP CONFIGURATION

ip dhcp excluded-address 192.168.10.1 192.168.10.9

ip dhcp excluded-address 192.168.20.1 192.168.20.9

ip dhcp excluded-address 192.168.99.1 192.168.99.9

ip dhcp pool STUDENTS

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

lease 1

ip dhcp pool STAFF

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

lease 1

ip dhcp pool GUESTS

network 192.168.99.0 255.255.255.0

default-router 192.168.99.1

dns-server 192.168.99.2

lease 0 4

end

write memory