LAB REPORT: NETWORK CONFIGURATION & SECURITY VERIFICATION

Student Name: Nimra Shabbir

Course: BS Computer Science

1. Project Objective

The main objective of this lab is to configure a secure network infrastructure with multiple VLANs (Students, Staff, and Guests), implement Dynamic Host Configuration Protocol (DHCP) IP pooling, enforce security restrictions using Extended Access Control Lists (ACLs), and apply Quality of Service (QoS) rate-limiting on guest traffic.

2. Network Configuration Script (Router)

! --- DHCP POOL CONFIGURATION ---

ip dhcp pool STUDENTS

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

lease 1

ip dhcp pool STAFF

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

lease 1

ip dhcp pool GUESTS

network 192.168.99.0 255.255.255.0

default-router 192.168.99.1

dns-server 192.168.99.2

lease 0 4

! --- ACCESS CONTROL LISTS (ACL) ---

ip access-list extended GUEST_ACL

permit udp any any eq 67

permit udp any any eq 68

permit udp 192.168.99.0 0.0.0.255 host 192.168.99.2 eq 53

permit tcp 192.168.99.0 0.0.0.255 host 192.168.99.2 eq 80

3. Verification & Screenshots

A. Guest Isolation Verification (Ping Drop)

To verify that guests are successfully restricted from accessing internal VLANs, an ICMP (ping) test was conducted from an Ubuntu guest cloud machine towards the internal subnet (192.168.10.10).

- Results: The router successfully blocked the traffic, returning a "Packet filtered" message with 100% packet loss. This proves the GUEST_ACL is functioning properly.

B. Bandwidth Management & Rate Limiting

To ensure fair distribution of network resources, a traffic policy was applied to the Guest network interface to limit network speeds to a maximum bandwidth cap of 5 Mbps.

4. Conclusion

All network requirements were successfully implemented and validated. The DHCP pools correctly assign IPs dynamically, the extended ACL effectively isolates guest traffic from critical internal subnets, and the bandwidth rate-limiting successfully controls network traffic overhead.